| PRODUCT NAME: | VANGO II | HMIS CODES: | H | C | F | R | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT CODE: | 7175 |  | 1 |  | 1 | 0 | B |

WHMIS CLASS: Not a Controlled Product

==================  SECTION I  -  MANUFACTURER IDENTIFICATION  =================

MANUFACTURER'S NAME: ATCO International

ADDRESS            : 2136 Kingston Court

Marietta, GA  30067-8902

INFORMATION PHONE  : (770)424-7550                      DATE PRINTED: 9/22/2006

PREPARED BY: MAURICIO A. AMAYA / LABORATORY MANAGER

REVISION DATE: 11/20/2003

==========  SECTION II  -  HAZARDOUS INGREDIENTS/SARA III INFORMATION  =========

| REPORTABLE COMPONENTS | CAS NUMBER PEL TLV |  |  | WEIGHT PERCENT |
| --- | --- | --- | --- | --- |
| n-METHYL PYRROLIDONE | 872-50-4 | ND | ND | 15-25% |
| DIMETHYL GLUTARATE | 1119-40-0 | ND | ND | 50-60% |
| DIMETHYL SUCCINATE | 106-65-0 | ND | ND | 15-25% |
| DIMETHYL ADIPATE | 627-93-0 | ND | ND | 5-15% |

===============  SECTION III  -  PHYSICAL/CHEMICAL CHARACTERISTICS  ============

| BOILING RANGE: | ND |  | SPECIFIC GRAVITY (H2O=1): |  | 1.08 |
| --- | --- | --- | --- | --- | --- |
| VAPOR DENSITY: | ND |  | EVAPORATION RATE: | ND |  |
| DENSITY: | 9.0 lb/gl |  | MATERIAL V.O.C.: | 15-20% |  |

SOLUBILITY IN WATER:	PARTIALLY SOLUBLE.           pH (Concentrate): NA

APPEARANCE AND ODOR:	PURPLE TOWEL.                pH (Diluted): NA

ODOR:  MILD ODOR

=================  SECTION IV  -  FIRE AND EXPLOSION HAZARD DATA  ==============

OSHA FLAMMABILITY CLASSIFICATION: CLASS IIIA

FLASH POINT: >200 (Liquid only) 		    METHOD USED: TCC

FLAMMABLE LIMITS IN AIR BY VOLUME- LOWER: ND        UPPER: ND

EXTINGUISHING MEDIA: Water fog, Carbon dioxide, Dry chemical, or Foam.

SPECIAL FIREFIGHTING PROCEDURES: No special firefighting equipment is needed. However, a self contained breathing apparatus and protective clothing should be worn in fighting fires involving chemicals.

UNUSUAL FIRE AND EXPLOSION HAZARDS: None.

========================  SECTION V  -  REACTIVITY DATA  =======================

STABILITY: Product is stable under normal conditions.

CONDITIONS TO AVOID: Open flame, sparks, or high temperature.

INCOMPATIBILITY (MATERIALS TO AVOID): Acids, alkalies, oxidizing materials.

HAZARDOUS DECOMPOSITION OR BYPRODUCTS: Thermal decomposition and burning may produce carbon dioxide and carbon monoxide.

HAZARDOUS POLYMERIZATION: Will not occur under normal conditions.

====================  SECTION VI  -  HEALTH HAZARD DATA  =======================

PRIMARY ROUTES OF ENTRY:  Inhalation  X_       Eye Contact _X_

Skin Contact _X_     Ingestion ____

EXPOSURE LIMIT: Refer to Section II for PEL / TLV exposure limits.

ACUTE EFFECTS OF OVEREXPOSURE

INHALATION: May result in varying degrees of irritation to the tissues in the respiratory tract.

EYE CONTACT: May cause eye irritation.

SKIN CONTACT: Can cause irritation upon prolonged or repeated exposure.

SKIN ABSORBTION: Can be irritating to skin

INGESTION: Can cause mouth , throat, and gastrointestinal tract irritation, nausea, vomiting, and diarrhea. Seek medical attention as required.

CHRONIC EFFECTS OF OVEREXPOSURE: Chronic effects and medical conditions aggravated by overexposure to this product have not been established. Unnecessary exposure to this product or any chemical should be avoided.

CARCINOGENICITY:  NTP CARCINOGEN:___  IARC MONOGRAPHS:___  OSHA REGULATED:___

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY OVEREXPOSURE: No additional effects of overexposure to this material can be found from available information.

EMERGENCY AND FIRST AID PROCEDURES

INHALATION: Remove to fresh air.

EYE CONTACT: Immediately flush eyes with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists get medical attention immediately.

SKIN CONTACT: Wash exposed area thoroughly with soap and water. Consult a physician if irritation develops.

INGESTION: NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON. If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. GET MEDICAL ATTENTION IMMEDIATELY.

===========  SECTION VII  -  PRECAUTIONS FOR SAFE HANDLING AND USE  ============

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: Stop source of spill.  Sweep solid material into a  suitable container and dispose of in a manner consistant with applicable regulations.

WASTE DISPOSAL METHOD: Dispose of in accordance with applicable Federal, State, and local regulations.  Under RCRA regulations, it is the responsibility of the product user to determine, at the time of disposal, whether a material is hazardous.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORING: Do not store above 120 F. Keep closure tight and container upright to prevent leakage. Store drum out of sunlight and away from heat, sparks, or flame. Store only in well-ventilated areas.

OTHER PRECAUTIONS: Follow label instructions.  KEEP OUT OF REACH OF CHILDREN.

===================== SECTION VIII  -  PERSONEL SAFETY MEASURES  ================

RESPIRATORY PROTECTION: None required in excess of normal ventilation.

VENTILATION: Local mechanical exhaust to provide normal ventilation.

PROTECTIVE GLOVES: Impervious gloves recommended.

EYE PROTECTION: Use safety eyewear designed to protect against splash of liquids.

OTHER PROTECTIVE CLOTHING OR EQUIPMENT: None required.

WORK/HYGIENIC PRACTICES: Wash thoroughly after handling.

=====================  SECTION IX  -  REGULATORY INFORMATION  ==================

TOXIC SUBSTANCES CONTROL ACT (TSCA): All chemical components in this product are

listed on the TSCA Inventory List, or are exempt from listing.

SARA Section 312 HAZARD CATEGORIES:   Fire Hazard___  Reactivity Hazard___

Acute Health Hazard_X_  Chronic Health Hazard___  Pressure Hazard___

# SARA Section 313 REPORTABLE COMPONENTS: (See Section II, If Applicable)

=====================  SECTION X  -  SHIPPING INFORMATION =====================

DOT SHIPPING NAME: ORM-D Consumer Commodity

DOT HAZARD CLASS: Not Applicable

UN NUMBER: Not Applicable

DOT PACKING GROUP: Not Applicable

SPECIAL SHIPPING INFORMATION: Not Applicable

===============================   DISCLAIMER   =================================

The information on this Material Safety Data Sheet represents our current available

data and best opinion as to the proper handling in use of this product under normal

conditions. Any use of this product which is not in conformance with this Data Sheet or which involves using this product in combination with other materials, or

processes is the sole responsibility of the user.